# Allo Dataflow Simulator - Profiling Results (standard simulator)

Host: 64 cores. Workload: parametric systolic GEMM (side x side x depth; PEs = (side+2)^2). Metrics are per warm simulation, build excluded, averaged over N runs; correctness checked vs numpy A@B. Source: simulator_profiling/ (scripts + RESULTS.md). These are the default (out-of-the-box) simulator numbers.

# What each column means

| Column | Meaning |
| --- | --- |
| config | Systolic array size, side x side x depth (e.g. 8x8x8). |
| PE | Number of processing elements = OS threads (after dead-corner removal). |
| PE/core | Oversubscription = PE count / 64 cores. >1 means more threads than cores. |
| build s | Time to compile the design to a JIT binary (df.build), seconds. |
| sim ms / +/-sd | Mean wall-clock time of one simulation, and run-to-run standard deviation. |
| busy cores | Average cores actually busy = (user+sys CPU time) / wall time. |
| sys_frac | Fraction of CPU time in the kernel = sys/(user+sys). Low = user-mode spin; high = scheduling/futex-bound. |
| ivcsw/sim | Involuntary context switches per simulation (OS forcibly preempting threads = wasted scheduling churn). |
| futex / yield / nsleep /sim | Syscall counts per sim (strace): futex (OpenMP lock/barrier), sched_yield (taskyield), nanosleep (usleep). Composition indicators, strace-perturbed. |
| futex %time | Fraction of syscall time spent in futex. |
| RSS MB | Peak resident memory. |
| max_abs_err | Correctness: max abs difference vs numpy A@B (tolerance 1e-3). |

# Timing sweep (standard simulator)

Two regimes, knee at the core count (64). Below ~1 PE/core the sim is user-mode spin-bound (sys_frac ~0.02); above it, it becomes ~80% kernel-scheduling-bound with millions of involuntary context switches. Build time dominates single-shot and grows with PE count. All configs PASS correctness.

| config | PE | PE/core | build s | sim ms | +/-sd | busy cores | sys_frac | ivcsw/sim | RSS MB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2x2x2 | 12 | 0.25 | 0.85 | 0.59 | 0.2 | 14.5 | 0.026 | 0 | 269 |
| 4x4x4 | 32 | 0.56 | 2.28 | 0.9 | 0.38 | 30.3 | 0.023 | 27 | 273 |
| 6x6x6 | 60 | 1.0 | 4.49 | 3.7 | 6.38 | 51.8 | 0.044 | 1566 | 280 |
| 8x8x8 | 96 | 1.56 | 7.66 | 21.8 | 26.3 | 55.0 | 0.794 | 409180 | 287 |
| 10x10x8 | 140 | 2.25 | 12.6 | 56.1 | 25.7 | 57.7 | 0.792 | 2000747 | 297 |
| 12x12x8 | 192 | 3.06 | 17.3 | 87.3 | 41.6 | 59.2 | 0.792 | 3657498 | 310 |
| 14x14x8 | 252 | 4.0 | 24.5 | 80.8 | 55.8 | 58.8 | 0.793 | 3269518 | 328 |
| 16x16x8 | 320 | 5.06 | 34.2 | 101.3 | 85.9 | 54.7 | 0.793 | 3138008 | 348 |

# Syscall breakdown (strace)

sys_frac and ivcsw are the clean (unstraced) values; syscall counts are strace-perturbed (composition, not magnitude). The kernel time is futex-dominated (81-95%) - OpenMP lock/barrier/park - not usleep.

| config | sys_frac | ivcsw/sim | futex/sim | yield/sim | nsleep/sim | futex %time |
| --- | --- | --- | --- | --- | --- | --- |
| 2x2x2 | 0.026 | 0 | 0 | 0 | 16 | 87% |
| 4x4x4 | 0.023 | 27 | 236 | 0 | 87 | 92% |
| 6x6x6 | 0.044 | 1566 | 0 | 1147 | 438 | 95% |
| 8x8x8 | 0.794 | 409180 | 0 | 3087 | 858 | 89% |
| 10x10x8 | 0.792 | 2000747 | 0 | 3194 | 1345 | 89% |
| 12x12x8 | 0.792 | 3657498 | 48 | 5498 | 2575 | 81% |
| 14x14x8 | 0.793 | 3269518 | 174 | 7370 | 3512 | 82% |
| 16x16x8 | 0.793 | 3138008 | 1883 | 8883 | 5503 | 82% |

# Deadlock detection (20s kill-timeout)

The simulator does NOT detect deadlock - it hangs forever (no error/diagnostic); only the correct control completes.

| case | design | outcome |
| --- | --- | --- |
| control | correct: 4 puts / 4 gets | COMPLETED (1.6s) |
| mismatch_get | consumer gets 1 more than produced | HANG -> killed at 20s |
| overfill_put | producer overfills depth-2 FIFO | HANG -> killed at 20s |
| circular | two kernels each get-before-put | HANG -> killed at 20s |

# Feedback / cyclic dataflow (15s kill-timeout)

Buffered feedback works correctly + deterministically WHEN PRIMED; the same cycle unprimed deadlocks. Priming/depth (the design), not the sim, decides.

| case | outcome |
| --- | --- |
| ring2_primed (2-kernel cycle) | COMPLETED - got=10=expected, deterministic (10,10,10) |
| ring3_primed (3-kernel cycle) | COMPLETED - got=20=expected, deterministic (20,20,20) |
| ring2_unprimed (same cycle, consume-first) | HANG (deadlock) - killed at 15s |

# Non-blocking non-determinism

Type-C test (producer 200 try_put, consumer 200 try_get, fixed count, no spin), same compiled sim + same input, run 30x: 23 distinct (put_ok, got) outcomes; both ranged the full 32..200. Same program + input -> 23 different answers, because non-blocking outcomes are decided by the OS scheduler (the simulator has no clock).